# How to Do Things with RVfpga

This document will provide some guidance about using RVfpga in different environments. For the avoidance of doubt, all directories are relative to the base directory holding all of the RVfpga information ../RVfpga

# Getting Started

Firstly, make sure you follow all the steps in Documents/RVfpga_GettingStartedGuide.pdf. This will allow you to install all of the tools associated with your platform. Additionally, go through all of the lab exercises in Labs/ up to and including Lab05 – at this point you will have re-built the FPGA bit file for RVfpganexys.

## Platform

There are three basic platforms that you might use:

Win;
MacOS;
Linux

Of the three the one that works in the most integrated manner is Linux because a) all the tools run under Linux and b) Linux is the main platform used in the Getting Started guide (Win, and MacOS) are side-shows really.

However, even in you only have access to a Win/MacOS machine you can (under certain circumstances run everything in a Linux environment – using VirtualBox (a VM environment). To do this, however, your computer must have circa 16Gb of RAM. Vivado, running under Linux, will need at least 6Gb of RAM and the amount of RAM you can allocated to Linux in VirtualBox should probably be less than 50% of the physical RAM in your system – leave too little RAM and the system won’t run properly.

To this end there is a VM appliance on the RVfpga Google folder, Ubuntu20_Windows.ova. This can be imported into VirtualBox. Any VM used should have circa 100Gb of disk space and access to at least 8Gb of RAM.

The issues you will face will depend on your Operating System, as follows:

Linux:

none of which I am aware. However, given that Vivado 2019.2 is needed you will encounter problems using the SelfExtracting application - it will report it is too old. You can download a single file repository - this downloads a zip file and when you extract it, the tools to install Vivado are in the extracted files.

Win:

Vivado runs on Win (make sure you use the exact version specified in the Getting Started document – co-incidently, this is the version you can access on the managed desktop). The only real problem is that the driver used to download bitstreams to the FPGA from Vivado is different to the drivers used by Visual Studio Code. This being the case, use Vivado to generate the bitstream (RVfpganexys.bit) but use Visual Studio code to upload the bitstream (and object code) to the FPGA board. Given that Vivado 2019.2 is needed you will encounter problems using the SelfExtracting application - it will report it is too old. You can download a single file repository - this downloads a zip file and when you extract it, the tools to install Vivado are in the extracted files.

Vivado will not run the rvfpgasim simulation. There are issues with Verilog version (i.e. some .v files actually need to be recorded as SystemVerilog files). Additionally, some of the defines in common_defines.vh (which is set as a global include) do not seem to be seen in a few cases (?).There are places where `default_nettype needs to be set to wire to avoid errors.  Finally, there are places where verilog statements that have no simulation implementation in Vivado are used. These are guarded by `ifndef XSIM and defining XSIM sidesteps this issue for simulation. However, when I had addressed all these problems, the final error I got was that I did not have enough memory to run the simulation (16Gb). So, using Vivado for simulation is problematic.

You will have problems re-compiling Verilator (the simulator). This needs to be done using GCC10 and Cygwin (a popular linux terminal emulator) uses GCC11 and this causes problems (I cannot see how to install GCC10 on Cygwin).

MacOs:

You should be able to recompile Verilator (I certainly could in a terminal window). Check the version of GCC installed (gcc –version) to be sure. You may have problems with newer OSs.

Vivado does not run on MacOs at all so another solution needs to be found.

Some of the steps in installation require other things to be installed e.g. HomeBrew (search “install homebrew mac” to find instructions about how to do this).

## Recommendations

If you can, set up a big enough Linux VM running under VirtualBox and do everything under Linux. Go to https://www.virtualbox.org/wiki/Downloads for downloads. Once you’ve installed VirtualBox you can either download and import the appliance on the RVfpga google folder or create an Ubuntu VM (see https://ubuntu.com/tutorials/how-to-run-ubuntu-desktop-on-a-virtual-machine-using-virtualbox#1-overview).

If the wrong version of GCC is installed then you can apt get/update/install to add support. Check https://askubuntu.com/questions/1192955/how-to-install-g-10-on-ubuntu-18-04 for instructions.

For information, for the Ubuntu appliance in the Google Folder, the su password is rvfpga2022

If you run Win then still install Ubuntu in VirtualBox but use it primarily to run Verilator.

If you run MacOs then either install Ubunutu in VirtualBox and use it to run Vivado or, indeed, run Vivado on another Win machine.

In cases where you are running VirtualBox, you should set up a Shared folder between the host machine and Linux running in VirtualBox. You can put your entire RVfpga directory inside this shared folder and it can then be accessed from your basic machine and Linux running in VirtualBox. So, for example, if you are running a Win computer, you can access this shared folder from Vivado, create your project in it, etc. If you want to run Verilator then you can open the Linux VM, recompile Verilator and run it (from Linux). This will generate a vcd (value change dump) file that can be viewed within GTK_wave.

Unfortunately, there is no single solution (other than Linux) that works best. It will all depend on your individual circumstances.

# Simulation

In this regard, there are two top-level modules for RVfpga:

rvfpganexys.sv – this is the top-level module for use when downloading your design onto an FPGA.

rvfpgasim.v – this is the top level module for use in simulation.

The reason is that rvfpganexys assumes that things like RAM are provided outside of the FPGA on the nexys A7 board. rvfpgasim has to provide memory models to emulate this memory within the top-level module.

It is important to realise (regardless of whether you’re using the FPGA or a simulation) that the memory used by the R-V needs to be loaded with the object code compiled from your C code. Otherwise, the code will not be executed.

## FPGA

When the R-V is running on the FPGA, the rvfpganexys.bit file is used to load the R-V implementation/configuration on to the FPGA via its JTAG port. Once it’s there and the R-V is still being held in reset, the JTAG port can then be used to load the object code (check out src/OtherSources/bscan_tap.sv – this is a JTAG test action point controller that allows the JTAG port(s) to generate peeks and pokes into the R-V’s memory using the dmi bus).

## Simulation

When the R-V is running in simulation, there is a section of Verilog in rvfpgasim.v that is responsible for doing this. Look at the code block beginning at L60 of this file. This is an initial block (which runs when the simulation starts). This code uses some Verilog functions that communicate back with the host computer e.g. $test$plusargs() and $value$plusargs(). These test for/return the value of an argument associated with the simulator. So, essentially, if the argument “jtag_vpi_enable” does not exist then get the value of the “ram_init_file” parameter – this will be the path to a file holding a hex dump of the object code. The $readmemh() function will then load the contents of this file (as 64 bit hex values) into an array of 64 bit numbers held in a component called ram.ram.mem. If you trace through the code, this is the memory model for the external RAM at 0x00000000.

Following this there is a similar block of Verilog code that can be used to load the memory used to model the boot ROM. This is a small block of memory at address 0x80000000. However, if no bootload ROM code file is specified then the initial block sets address 0x80000000 to 0x67 and this corresponds to JMP 0. That is, when it starts executing it executes the instruction at address 0x8000000, which is JMP 0, and the next instruction is fetched from address 0 (which is where your object code has been put).

If you are using Verilator, then this is all built into the build process and when vrfpgasim.exe runs, the object code will be loaded into the memory before the Verilator releases the simulated R-V from reset. Notice at L25 of rvfpgasim.v, where the testbench module is defined. If VERILATOR is defined then the testbench has a bunch of inputs and outputs which are captured by a module that is part of Verilator (and not your design). This module supplies clk, reset, a JTAG port, and a serial port. This allows Verilator to control your design, load stuff in via JTAG and monitor the serial port. If VERILATOR is not defined then the JTAG port and serial port are not connected, a clk signal of 50MHz is generated and rst goes inactive after 100ns. For more information, look at tb.c in ./verilatorSIM. This is the main c file and drives/connects rvfpgasim and the jtag block.

## Build Directory

If you create a project directory from within Visual Studio Code (e.g. MyProject) then there is (on a Mac a hidden) directory: MyProject/.pio/build/swervolf_nexys/ directory. Inside here you will find files produced by the code build: firmware.bin, firmware.elf, firmware.dis and firmware.vh. firmware.dis is the disassembly of the code generated. Look at this and compared the code values (2nd column) against the data in firmware.vh – they are the same but firmware.vh can be read by $readmemh(filename, destination_regs). You will also find trace.vcd here. This is the file created by Verilator when it is executed. Double click on this and GTK_wave should open it allowing you to see the waveforms generated during simulation.

# Creating Your Design

Before you start using the R-V design at all you should produce a stand-alone design of your ‘component’ within Vivado that you can develop and test independently. However, you should always consider how your design will then fit into the memory space of the R-V and how the R-V will use it. Remember, the R-V can really only do a few things: it can read/write 32 bit memory locations (which you will organize into a set of ‘registers’ according to your needs). It can respond to an Interrupt, causing it to call an associated function. If your component does something that periodically requires the R-V’s intervention, it is worth considering the use of an interrupt to signal when something interesting has happened with your component. This precludes the R-V from spending a lot of time checking to see if something interesting has happened.

Once you have a design that is working, you should then put it inside a wrapper that allows your component to interact sensibly with the R-V via the Wishbone bus. Look at the wishbone specification at https://cdn.opencores.org/downloads/wbspec_b3.pdf). Clearly, wishbone is yet-another-bus that behaves similarly to a lot of other buses but, for example, it is nowhere near as complicated (or flexible) as AXI. The easiest way to find out how to implement the interface from wishbone to your component in the wrapper is to look at src/SweRVolfSoC/Peripherals/gpio/gpio_top.v and see how the GPIO port component does it. Try to replicate the logic for address decoding, selection, timing of transfer, and signal generation. If you do this then creating a wrapper to meet your specification will be straightforward. In this regard, when you want to put your component design into the R-V you should create a new directory e.g. MyComponent under src/SweRVolfSoC/Peripherals/ and put all of your RTL code in there. Keep a copy of the original src directory (your golden copy if you fatally damage your RTL code) and only make changes to a separate working copy (in src/).

However, putting your component code into a sub-directory is not enough. It needs to be incorporated into your design and to do this you will need to alter the RVfpga RTL.

The first thing to alter is src/SweRVolfSoC/swervolf_core.v – this is the top level of the R-V ‘chip’. If you look at this Verilog file, the swervolf_core component contains a number of other components:

| Component | Purpose |
| --- | --- |
| swervolf_eh1 | the R-V processor |
| axi2wb | a component that converts the IO AXI bus coming out of the processor to a wishbone bus (wb_x_io) |
| wb_intercon | A component that steers the wb_io bus in a number of different directions (based on address decoding) and allows peripherals to be attached at its outputs. |
| Peripheral components | A bunch of peripheral components attached to the outputs of wb_mux: boot ROM; syscon; spi uart16550, gpio_module; ptc; spi2 |

So, this is where you would instantiate your component (along with the other peripheral components). You component wrapper will have, at least, a wishbone slave interface and this should be connected back to the wb_intercon (if your component has other pins that you want to come out of the design to the FPGA board then you will need to burrow these pins out of all the surrounding components all the way up to rvfpganexys – inputs/outputs to this component will be pins on the FPGA).

You will note that the naming convention for each of the sets or wires connected to the wishbone interfaces on the peripheral components is well defined e.g. for the gpio you have: wb_m2s_gpio_X and wb_s2m_gpio_X (where m2s is for wires going from the mux to the device, s2m is for wires going in the other direction and X is the wire function: cyc, adr, dat, we, stb, ack, err).

So:

Step 1: instantiate your component in swervolf_core.v using a wishbone bus where the naming convention is wb_m2s_mycomp_X, etc. where mycomp is the name of your component.

If you look through swervolf_core.v you will notice two things:

wb_intercon does not exist in the file – which is worrying because it should;

None of the wb slave buses e.g. wb_gpio_X are declared in the file. Again, this is worrying because they should be.

This is because there is a statement on L145:

`include “wb_intercon.vh”

This includes a Verilog header file.

If you look in wb_intercon.vh you will see that all of these undefined wires for the slave wishbone buses are declared here.

Step 2: edit wb_intercon.vh to add the wishbone bus for your component. Hint: copy and then paste the wire definitions for GPIO and just change gpio to mycomp in the copied set of wire definitions.

Further down at wb_intercon.vh at L104 there is a component instantiation for wb_intercon. However, looking down the port mappings, there is no mention of your new bus. So…

Step 3: add a set of port mappings (inputs/outputs) for your new wishbone bus. Hint: find the port mappings for GPIO, copy these and past ethem at the end after the SPI block. Be careful, there are two )) at the end of this set of signals – paste between the two ). Then change gpio to mycomp in the wires mapped to the ports and the port names in the copied block of text.

Now, the instance of wb_intercon has an extra output port but, unfortunately, the instance of wb_intercon now no longer matches its definition.

To solve this problem, open wb_intercon.v – this is where the component is defined. Looking down the list of inputs/outputs for this component, there is no wishbone bus for your component. So ..

Step 4: add a set of additional inputs/outputs to the definition of wb_intercon. Hint: copy the input output port definitions for the wb_gpio bus and paste them at the end after the SPI port. Again, there is ); after the last spi signal so paste just before );. Then change gpio to mycomp in all of the copied inputs and outputs. Additionally, make sure there is a comma after the last spi signal definition and make sure there is no comma after the last mycomp signal otherwise this will be a syntax error.

Now you have a component connected to a set of outputs from the wb_intercon component but the logic for steering memory accesses to the port needs to be created. Fortunately, this is almost zero effort.

The actual steering is done by a component inside wb_intercon that is called wb_mux.  You will note that there are 3 parameters associated with this component: num_slaves, MATCH_ADDR and MATCH_MASK. num_slaves is 7 and both MATCH_ADDR and MASK are the concatenation of 7, 32 bit numbers. Additionally, moving down the instantiation, there is a set of generic wishbone ports (adr_o, dat_i/o, sel_o, etc.). Similarly each of these is a concatenated set of signals where each column of signals defines one of the slave wishbone ports. Again there are 7 columns of ports in total.

Now num_slaves defines the number of slave ports – 7 in this case;

Each 32 bit word in MATCH_ADDR defines the base address of the wishbone slave port (relative to 0x80000000) for that column position. So, MATCH_ADDR in the second column is 0x00001000 and this column is where the sys wishbone port sits (this connects to the syscon component). This means that the base address in memory where syscon lives is 0x800001000. MATCH_MASK does a complementary job and defines which bits of the incoming address are compared against MATCH_ADDR (equality steers the incoming access along the corresponding, selected port).

For the second column the match value is 0xffffffc0 and this means that any incoming transfer whose address matches 0x8000010{00xx}x will be steered to the bus in the second column position. {} in this case is to split the hex digit into a 4 bit binary value. This means that any access to an address in the range 0x80001000-0x80000103f will result in the access being steered to the syscon component. Looking at the list of existing components we have:

| Device | Start Address | End Address | Space occupied |
| --- | --- | --- | --- |
| Boot ROM | 0x80000000 | 0x80000fff | 0x1000 |
| syscon | 0x80001000 | 0x8000103f | 0x40 |
| spi flash | 0x80001040 | 0x8000107f | 0x40 |
| spi accel | 0x80001100 | 0x8000113f | 0x40 |
| ptc | 0x80001200 | 0x8000123f | 0x40 |
| gpio | 0x80001400 | 0x8000143f | 0x40 |
| uart | 0x80002000 | 0x80002fff | 0x1000 |

Note, none of these address ranges overlaps (which would be an error) and there are gaps between some of these components e.g. the address range 0x80001080-0x80001fff is unused. Nor, additionally, is it clear why the UART needs 0x1000 memory locations. However, this tells us what we need to know:

Step 5: increase the num_slaves parameter to 8.

Step 6: add new 32 bit numbers to the end of the concatenation for both MATCH_ADDR and MATCH_MASK. So, for example, if you wanted your component to sit at 0x80003000 and occupy a space of 0x80 bytes then the new value at the end of MATCH_ADDR should be 32’h00003000 and the new value at the end of MATCH_MASK should be 32’hffffff80.

Step 7: using the new input/output names for your components wishbone bus, add each name to the end of the concatenation for each of the wishbone sets of signals. For example, add wb_mycomp_adr_o to the end of the concatentation mapped to the wbs_adr_o port, etc.

Once this has all been done, your component is now integrated within RVfpga.

# Adding your Component files to the design

If you are working within Vivado you will need to add existing sources into your project adding all of the Verilog files associated with your component.

For Verilator you need to do some work. Verilator builds the simulation based on information is finds in verilatorSIM/swervolf_0.7.vc (essentially this looks like a Verilog configuration file). If you look at this file then at the top you will see a set of +incdir+ command. In particular, if you look down this list you will see a +incdir+ instruction for each of the peripheral components e.g.

+incdir+ ../src/SweRVolfSoC/Peripherals/gpio

This would tell a simulator or synthesis tools that it needs to look in this directory.

Step 8: add an incdir instruction (near the other peripherals) for your component e.g. +incdir+ ../src/SweRVolfSoC/Peripherals/mycomp

Additionally, there is a corresponding of -CFLAGS -I commands with the same set of directories. This adds an include directory for the C complier (I’ve no idea why/if this is necessary but is used when the verilator executable is built):

Step 9: add an -CFLAGS instruction (near the other peripherals) for your component e.g. -CFLAGS – I  ../src/SweRVolfSoC/Peripherals/mycomp

Below these commands is a long list of files and these are all of the Verilog files that need to be pulled together to create a full design. If you look towards the bottom you will see pathnames for all of the components’ Verilog files e.g. ../src/SweRVolfSoC/Peripherals/gpio/gpio_top.v

Step 10: near the bottom of this file (next to the gpio_top path, for example) add the path(s) to all the Verilog files that define your component e.g. ../src/SweRVolfSoC/Peripherals/mycomp/mycomp_top.v

Now when you rebuild Verilator it should be aware of your component and incorporate it into the simulation.

# Conclusion

These notes describe some of the issues (and resolutions) that you may encounter. They may not reflect the issues that you find. If you do find any additional issues and/or there are problems with what is written in this document then let me know and I will update the guidance.